Quick Reference Guide: Original Prusa MINI+

SAFETY FIRST (READ BEFORE USE)

- General Safety: Never leave the printer unattended during the first layer.

- Pinch Hazard: Keep hands clear of the moving X, Y, and Z axes during operation.

- Heat Hazard: The nozzle can reach 280°C and the heatbed 100°C. Do not touch until cooled.

- Clothing: Tie back long hair and remove loose clothing or jewelry.

Uploading Files:

- Open PrusaSlicer, load your model (how to do that)

- Design/Print alterations like: size, support, infill, brim,etx.

- Select the filament color/type/brand that you will be using.

- Determine print speed/quality

- Slice your model in PrusaSlicer using the Mini+shaper profile.

- Export the .gcode file to a USB drive.

Power-On Sequence:

- Insert the USB drive into the printer’s port.

- Check that the top and bottom of the removable build plate is clean and free of debris.

- Switch the power button (on the right side) to ON.

- Wait for the LCD screen to initialize.

- Load the filament color/type/brand that you selected in PrusaSlicer

- Select the file from the menu and press the knob to start.

BED LEVELING & FIRST LAYER

- Mesh Bed Leveling: The printer runs this automatically before every print to ensure the bed is flat.

- Z-Calibration: If the first layer is too high (gaps) or too low (squished), adjust the Live Z setting during the first layer print.

TROUBLESHOOTING

Machine Will Not Start

- Check Power: Verify the power cable is securely plugged into the printer and the wall outlet.

- Check Switch: Ensure the main power switch on the side of the electronics box is in the ON position.

- Check Fuse: If the screen remains dark while switched on, inspect the fuse located in the power entry module.

Print Not Adhering to Bed

- Clean Surface: Wipe the magnetic steel sheet with 90% Isopropyl Alcohol to remove skin oils or dust. Check under the removable steel sheet for any debris.

- Check First Layer: Ensure the nozzle height is correct; if it is too high, your extruded filament will look more like round string. Use the Live Adjust Z menu to lower the nozzle during the first layer. Ideally, the filament should appear flattened. BEST PRACTICE: Run a First Layer Calibration – located “here”

- Check Bed Temperature: Confirm the bed temperature matches the filament manufacturer's requirements (Generalized e.g., 60°C for PLA, ? °C for PETG, 100°C for ABS).

Filament Not Extruding

- Check Temperature: Ensure the nozzle is heated to the correct melting temperature for the loaded material. Generalized e.g., 215°C for PLA, etc

- Check Tension: Inspect the extruder idler screw; if it is too loose, the gears will not grip the filament. The top of the screw is relatively flush with the plastic assembly.

- Check for Clogs: If the motor is clicking, perform a "Cold Pull" or use a thin needle to clear the nozzle tip. Or get help. Caution: The Nozzle hottest part!

Loading Filament

- Filament from Spool: Filament not reaching the filament drive motor. Apply pressure towards filament sensor to ensure the the filament reaches the motor.

X/Y/Z Axis Movement Error

- Check Obstructions: Ensure there are no zip ties, debris, or cables blocking the path of the toolhead or the bed.

- Check Belt Tension: Verify that the belts are tight and not slipping on the motor pulleys.

- Check Lubrication: If movements are loud or jittery, apply a small amount of machine lubricant to the smooth linear rods.

Poor Print Quality (Stringing or Blobs)

- Check Filament: Moisture in the filament can cause stringing; try a fresh spool or dry the current one.

- Check Slicer Settings: Ensure the "Retraction" settings in your slicer software are optimized for the MINI+.

- Check Nozzle: Inspect the nozzle for wear; a worn-out nozzle will lead to inconsistent extrusion.

Quick Reference Guide: SawStop 10" Contractor Saw

SAFETY FIRST (READ BEFORE USE)

- Brake System: This saw is equipped with an automated brake that stops the blade on contact with skin. Never disable this safety feature.

- Eye/Ear Protection: Always wear safety glasses and hearing protection.

- Material Prep: Ensure wood is free of nails, staples, or excessive moisture (which can trigger the brake).

- Push Sticks: Always use a push stick for rips narrower than 6 inches.

BASIC OPERATION

Power-On Sequence:

- Turn on the Main Power switch.

- Wait for the system self-test (Green light should be solid).

- Pull the Start/Stop paddle to start the blade.

Adjustments:

- Blade Height: Use the front handwheel to set the blade roughly 1/8" above the material.

- Blade Tilt: Use the side handwheel to set the desired bevel angle.

TROUBLESHOOTING

- Red Light Flashing: The brake system has detected an error or a triggered cartridge. Power down and inspect.

- Blade Won't Start: Check that the thermal overload reset button has not tripped and that the blade is not touching the guard.

Quick Reference Guide: Bambu Lab P1S

SAFETY FIRST (READ BEFORE USE)

- Ventilation: Ensure the internal fan and room ventilation are active, especially when printing ABS or ASA.

- Automated Motion: The toolhead moves at extremely high speeds. Keep the front door and top glass closed during operation.

- Lid Safety: Opening the door does not automatically stop the print; use the "Pause" command on the screen or app.

BASIC OPERATION

Power-On Sequence:

- Flip the power switch on the rear of the machine.

- The screen will light up and the machine will perform a homing routine.

Uploading Files:

- Use Bambu Studio to slice your files.

- Send the file via Wi-Fi (Cloud) or save to a MicroSD card.

- Select "Print" from the interface or the Bambu Handy app.

TROUBLESHOOTING

- Filament Jam: If the AMS (Automatic Material System) fails to load, check for brittle filament or broken pieces in the PTFE tubes.

- First Layer Issues: Ensure you have selected the correct build plate type (Textured PEI vs. Cool Plate) in the slicer.

Quick Reference Guide: OMTech AF3555-130W-CO2

SAFETY FIRST (READ BEFORE USE)

- Supervision: NEVER leave the laser unattended while running; stay in the room and watch the job from start to finish.

- Personal Safety: Tie back long hair, remove loose clothing, and avoid dangling jewelry.

- Fire Safety: Laser cutting can ignite materials; be always prepared.

- Emergency Action: If a small flame occurs, STOP the job and use a water spray bottle; for out-of-control fires, use an ABC fire extinguisher and evacuate if necessary.

- Banned Materials: Never cut PVC/Vinyl (releases chlorine gas), Polycarbonate (Lexan), ABS plastic, Fiberglass, or unknown foams.

BASIC OPERATION

Power-On Sequence:

- Turn ON water chiller.

- Turn ON exhaust fan.

- Turn ON air assist.

- Turn ON main laser power.

- Power on the Ruida control panel and wait for system stabilization.

Auto Adjust (Focal Length):

- Place material flat on the bed.

- Move laser head over the material.

- Select Auto Focus / Auto Adjust on the control panel.

- The machine will automatically set the correct focal distance.

TROUBLESHOOTING

- Laser Will Not Fire: Check if the lid is fully closed, the water chiller is running with no alarms, and the E-stop is released.

- Water Flow Alarm: Check water levels and ensure hoses are not kinked; do NOT bypass the alarm.

- Smoke Not Clearing: Ensure the exhaust fan is ON and ducting is not blocked.

Quick Reference Guide: Haas Minimill / UMC-500

SAFETY FIRST (READ BEFORE USE)

- Enclosure Safety: Never bypass the door interlocks. The spindle and axes will stop if the door is opened during a cycle.

- Eye Protection: Safety glasses are mandatory at all times when the machine is powered on.

- Tooling: Ensure all tools are securely seated in the tool changer and the pull studs are tightened to spec.

- Clearance: Always "dry run" new programs with the Rapid Override turned down to check for potential tool crashes.

BASIC OPERATION

Power-On Sequence:

- Turn the main breaker (rear of machine) to the ON position.

- Press Power Up/Restart on the control pendant.

- The machine will home all axes (X, Y, Z, and A/B for the UMC-500).

Setting Offsets:

- Use the jog handle to move the tool to the top of the workpiece for the Z-offset.

- Use a probe or edge finder to locate the X and Y work coordinates.

- Input values into the Work Offset table (e.g., G54).

Troubleshooting

Machine Has Power but Won't Move

- Check E-Stop: Verify that the Emergency Stop button is not depressed.

- Door Interlock: Ensure the enclosure doors are fully closed; the machine will not move if the safety interlock is open.

- Restart Control: Power cycle the control panel to clear any temporary software hang-ups.

- Power Cycle: Turn off the entire machine, wait 30 seconds, and restart to reset the internal drives.

Spindle or Axis Alarm Active

- Read the Screen: Check the specific alarm code displayed on the control panel for a detailed description of the fault.

- Reset Alarms: Press the RESET button to clear non-critical errors.

- Way Lube/Air: Confirm that the air pressure is at the required levels and the automatic lubricator is not empty.

Cuts Not Accurate or Poor Finish

- Check Tooling: Inspect the tool for wear, chips, or dullness; ensure it is properly seated in the spindle.

- Verify Offsets: Re-check your tool and work offsets to ensure the machine knows the exact position of the material.

- Coolant Flow: Ensure the coolant is hitting the tool-workpiece interface to prevent overheating and tool deflection.

Tool Changer Error

- Clear Obstructions: Check for metal chips or debris that may be preventing the tool changer from moving freely.

- Air Pressure: Low air pressure often prevents the tool release piston from firing correctly.

- Manual Recovery: Use the machine’s "Tool Changer Recovery" mode to step through the cycle and clear the jam safely.

When in Doubt

- STOP the machine: Press the Emergency Stop immediately if you hear unusual noises.

- Power down safely: Shut the system down according to the standard sequence.

- Ask for help: Consult a supervisor or trained operator before attempting manual repairs.

- No Testing: Never "test" a fix while the machine is active or a program is running

Quick Reference Guide: Bridgeport Series I 2HP Mill

SAFETY FIRST (READ BEFORE USE)

- Spindle Brake: Ensure the spindle is at a complete stop before changing tools or touching the workpiece.

- Entanglement: Absolutely no gloves, loose sleeves, or jewelry when operating the manual mill.

- Clearance: Ensure the quill is locked or fully retracted when not in use.

- Clamping: Ensure the vise or workpiece is securely bolted to the table; use a T-handle wrench to tighten.

BASIC OPERATION

Changing Speeds:

- Note: Only change the variable speed dial while the motor is RUNNING.

- Shift between High and Low range only when the motor is STOPPED.

Power Feed (If Equipped):

- Check travel limits to ensure the table will not hit the end-stops.

- Engage the directional lever and adjust the feed rate dial slowly.

Troubleshooting

Spindle Will Not Start

- Check Power: Ensure the main disconnect switch is in the ON position.

- Emergency Stop: Verify the E-Stop button (if equipped) is released.

- Forward/Reverse Switch: Ensure the drum switch is fully engaged in either Forward or Reverse; if left in the middle, the motor will not receive power.

- Thermal Overload: If the motor is hot and won't start, the internal thermal overload may have tripped; allow it to cool and reset.

Machine Vibrating or Making Loud Noise

- Speed Range: Confirm the Hi-Low range lever is fully seated; a partially engaged gear will cause grinding noises.

- Drive Belt: Inspect the variable speed drive belt for fraying or flat spots.

- Tooling: Ensure the R8 collet is tightened properly with the drawbar; loose tooling causes severe chatter.

- Quill Lock: Ensure the quill lock is secure if you are not performing a drilling operation.

Poor Surface Finish (Chatter Marks)

- Check Sharpness: Replace dull end mills or inserts; dull tools increase cutting pressure and vibration.

- Spindle Speed: Adjust the variable speed dial—often, reducing the RPM will eliminate harmonic chatter.

- Workholding: Ensure the vise is bolted securely to the table and the workpiece is clamped tightly.

- Gib Adjustment: If the table has excessive "play," the gibs may need to be tightened by a supervisor.

Quill is Hard to Move or Sticking

- Lubrication: Apply a small amount of machine oil to the quill surface and move it up and down to distribute.

- Clock Spring: If the quill does not return to the top automatically, the clock spring may be broken or disconnected.

- Lock Engagement: Ensure the quill lock handle is completely released.

When in Doubt

- STOP the machine: Engage the spindle brake immediately if you hear a crash or unusual grinding.

- Power down safely: Turn off the motor before attempting to clear chips or adjust tooling.

- Ask a supervisor: Manual mills require precise tactile feedback; if it feels "wrong," ask for a trained operator’s assistance.

- Never "test" fixes: Do not attempt to adjust the variable speed dial unless the motor is already running.

Quick Reference Guide: Hobart IronMan 230 / Miller Welding Machine

SAFETY FIRST (READ BEFORE USE)

- PPE Mandatory: Always wear a welding helmet with the correct shade, flame-resistant gloves, and a leather apron.

- Ventilation: Ensure the exhaust system is ON or work in a well-ventilated area to avoid inhaling toxic fumes.

- Fire Hazard: Clear the area of all flammable materials (rags, paper, solvents) before striking an arc.

- Electrical Safety: Never weld in damp or wet conditions. Ensure the machine is properly grounded.

BASIC OPERATION

Power-On Sequence:

- Connect the ground clamp to the workpiece or welding table.

- Open the valve on the shielding gas cylinder (if using MIG/TIG).

- Switch the main power toggle to ON.

- Adjust voltage and wire feed speed (or amperage) based on the material thickness chart found inside the side panel.

Shut-Down Sequence:

- Turn OFF the welding machine power.

- Close the shielding gas cylinder valve completely.

- Trigger the gun (MIG) to bleed the remaining gas from the lines.

TROUBLESHOOTING

- Porous Welds: Check if the gas cylinder is empty or if there is a draft blowing away the shielding gas.

- Wire Nesting (Bird Nest): Stop immediately. Check the drive roll tension and ensure the contact tip is not clogged or melted.

Quick Reference Guide: DeWALT 12" Double Bevel Sliding Miter Saw

SAFETY FIRST (READ BEFORE USE)

- Hand Safety: Keep hands at least 6 inches away from the blade path. Never cross your arms when holding the material.

- Eye Protection: Safety glasses are always required.

- Material Support: Always use the fence. For long pieces, use external stands to prevent the material from tipping or binding.

- Wait for Stop: Never lift the saw head until the blade has come to a complete and total stop.

BASIC OPERATION

Power-On Sequence:

- Ensure the blade guard moves freely.

- Plug in the saw and turn on the integrated work light/laser line if equipped.

- Squeeze the trigger to start the motor. Allow the blade to reach full speed before contacting the wood.

Adjusting Angles:

- Miter: Unlock the miter handle and move the arm to the desired degree (0°, 22.5°, 45°).

- Bevel: Release the bevel lock lever at the rear of the saw to tilt the head for angled vertical cuts.

TROUBLESHOOTING

- Inaccurate Cuts: Check for sawdust buildup against the fence or under the miter plate.

- Motor bogs down: Ensure you are not forcing the cut too quickly; let the speed of the blade do the work.

Quick Reference Guide: Grizzly 17" 2 HP Bandsaw

SAFETY FIRST (READ BEFORE USE)

- Blade Guard: Adjust the upper blade guide to be no more than 1/4" above the material being cut.

- Tension: Do not operate if the blade is loose or tracking incorrectly.

- Push Sticks: Use a push stick for small pieces to keep fingers away from the blade.

BASIC OPERATION

- Adjust the blade guide height for your workpiece.

- Turn ON the dust collection system.

- Press the START button.

- Feed material slowly, avoiding excessive side pressure on the blade.

Troubleshooting

Machine Has Power but Won't Move

- Check E-Stop: Verify that the red Emergency Stop button is not depressed.

- Door Safety Switches: Ensure both the upper and lower wheel covers are completely closed and latched; the motor will not engage if the safety switches are open.

- Foot Brake: Ensure the foot brake is fully released and has not mechanically locked the motor.

- Power Cycle: Turn the main power switch to OFF, wait 30 seconds, and turn it back ON to reset the magnetic switch.

Blade Wandering or Inaccurate Cuts

- Check Tension: Verify the blade is tensioned correctly for its width using the tension scale on the upper wheel.

- Guide Bearings: Ensure the upper and lower guide bearings are adjusted to roughly 0.004" (the thickness of a piece of paper) from the blade.

- Blade Guard Height: Confirm the guard is set no more than 1/4" above the material.

- Dull Blade: If the blade requires excessive force to cut or wanders to one side regardless of setup, the teeth are likely dull and the blade must be replaced.

Excessive Vibration or Noise

- Blade Tracking: Turn the machine OFF and rotate the wheels by hand to ensure the blade is centered on the tires.

- Wheel Tires: Check for built-up sawdust or "pitch" on the rubber tires; clean with a stiff brush if necessary.

- Drive Belt: Inspect the V-belt for cracks, slipping, or debris.

- Fence Alignment: Ensure the fence is perfectly parallel to the blade miter slot.

Motor Bogs Down or Smokes

- Feed Rate: Slow down the feed rate; let the speed of the blade do the work without forcing the wood.

- Dust Collection: Ensure the dust collector is ON and the internal cabinet is clear; packed sawdust can create friction against the lower wheel.

- Incorrect Blade: Ensure you are using a blade with the correct TPI (Teeth Per Inch) for the thickness of the wood being cut.

When in Doubt

- STOP the machine: If you hear a loud "bang" (likely a snapped blade), immediately step back and hit the E-Stop or foot brake.

- Power down safely: Unplug the machine before opening wheel covers or performing any blade changes.

- Ask a supervisor: Bandsaws require precise "tuning"; if you are unsure about tracking or tension, ask for a trained operator's assistance.

- Never "test" fixes: Do not attempt to adjust guide bearings while the blade is in motion.

Quick Reference Guide: Roland CAMM-1 GS-24 Vinyl Cutter

SAFETY FIRST (READ BEFORE USE)

- Entanglement: Keep hands and lose clothing away from the moving carriage and blade.

- Power Safety: Do not operate if the machine emits smoke, burning odors, or unusual noises.

- Blade Safety: Do not touch the tip of the blade; it is extremely sharp.

BASIC OPERATION

Setup:

- Blade Height: Adjust the blade holder so the tip protrudes roughly with the thickness of a credit card.

- Load Material: Lift pinch rollers, insert vinyl (roll or piece), and align with the marks.

- Set Origin: Use arrow keys to move the blade to the starting point and press the [Origin] button.

Software:

- Import vector data (DXF, SVG, or AI) into Roland CutStudio.

- Perform a Test Cut to ensure the blade force (typically ~90 gf) is correct for your material.

TROUBLESHOOTING

Machine Has Power but Won't Move

- Check Loading Lever: Ensure the loading lever is fully raised to the "Secure" position; the carriage will not move if the material is not locked.

- Pinch Roller Alignment: Verify that the pinch rollers are positioned directly over the grit rollers (indicated by white marks); the machine will error if they are misaligned.

- Clear Obstructions: Check for scrap vinyl or debris stuck in the carriage path or under the cutting strip.

- Unusual Noise: If the machine emits unusual noises or burning odors, power down immediately and unplug the unit.

Vinyl is Slipping or Feeding Crooked

- Check Alignment: Ensure the vinyl is loaded straight and aligned with the guide lines on the front and back of the machine.

- Roland Roller Tension: Verify that the pinch rollers are clean and not covered in adhesive residue.

- Material Size: Ensure the material width is sufficient for the job; use the [Menu] key to re-measure the loaded piece if necessary.

Cuts Are Incomplete or Jagged

- Check Blade Force: If the blade is not cutting through the vinyl, increase the force setting (gf) on the control panel or in the software.

- Inspect Blade Tip: Check the blade for chips or broken tips; do not touch the tip directly as it is extremely sharp.

- Blade Extension: Ensure the blade is protruding roughly the thickness of a credit card from the holder.

- Protection Strip: Inspect the black Teflon protection strip under the blade path; if it is deeply grooved or scratched, it must be replaced to ensure even cutting pressure.

Software Communication Error

- Check USB Connection: Ensure the USB cable is securely connected to both the cutter and a high-speed port on the PC.

- Correct Driver: Verify that the "Roland GS-24" is selected as the active printer/cutter in CutStudio or your design software.

- Set Origin: If the machine starts cutting in the wrong place, ensure you have set the [Origin] point on the machine's control panel before sending the file.

When in Doubt

- STOP the machine: Turn off the power switch if the carriage behaves erratically.

- Power Cycle: Turn off the machine, wait 30 seconds, and turn it back on to reset the internal memory.

- Ask a supervisor: If you are unsure about blade depth or force settings, consult a trained operator to avoid damaging the protection strip.

- Never "test" fixes: Do not reach toward the carriage while the machine is actively cutting

Quick Reference Guide: Wazer (Desktop Waterjet)

SAFETY FIRST (READ BEFORE USE)

- High Pressure Hazard: Never attempt to touch the water stream or place hands under the cutting head while the high-pressure pump is active.

- Eye Protection: Always wear safety glasses to protect against abrasive splashback and water spray.

- Water Levels: Ensure the tank is filled to the correct level before starting to prevent pump damage.

- Abrasive Handling: Use only approved garnet abrasive; handle with care to avoid inhaling dust when filling the hopper.

BASIC OPERATION

Power-On Sequence:

- Turn on the water supply valve.

- Switch the main power button to ON.

- Check the abrasive hopper level and refill if necessary.

- Clear the cutting bed of any old material or debris.

Running a Job:

- Upload your design via the Wazer software (WAM).

- Secure your material using the provided clamps to prevent shifting.

- Ensure the nozzle height is set correctly using the provided feeler gauge.

- Press START on the control panel.

MAINTENANCE

- Abrasive Disposal: Regularly empty the used abrasive collection buckets.

- Nozzle Wear: Inspect the mixing tube/nozzle for wear every 10-20 hours of cutting time.

TROUBLESHOOTING

High-Pressure Pump Will Not Start

- Check Water Supply: Ensure the water source is turned ON and the inlet hose is not kinked. The pump will not engage without sufficient water pressure.

- Low Water Level: Check the main tank; if the water level sensor is not submerged, the machine will prevent the pump from starting to avoid damage.

- E-Stop & Power: Verify the Emergency Stop is released and the machine is connected to a dedicated 20A circuit.

- Software Interlock: Ensure the WAM software has "Handshaked" with the machine and there are no active software warnings.

Abrasive (Garnet) Is Not Flowing

- Check the Hopper: Ensure the abrasive hopper is at least 1/4 full.

- Moisture in Hopper: If the garnet is damp, it will "clump" and stop feeding. Empty the hopper and refill with dry, sifted abrasive.

- Clogged Orifice: Inspect the abrasive feed line for obstructions. If the nozzle has a "backflow" (water entering the abrasive line), the orifice is likely clogged.

- Pinch Valve: Verify the internal pinch valve is operating correctly and not stuck in the closed position.

Poor Cut Quality or Nozzle Dragging

- Nozzle Height: Ensure the nozzle is set exactly 1.5mm to 2mm above the material using the provided height gauge. If it is too high, the kerf will be wide and tapered.

- Worn Mixing Tube: Mixing tubes are consumables; if the exit hole is no longer perfectly round or has become significantly larger, replace the tube.

- Material Shifting: Ensure the material is clamped securely to the bed. The pressure of the water jet can move unsecured sheets, causing ruined cuts.

- Incorrect Material Settings: Verify that the correct material type and thickness are selected in the WAM software.

Tank Overflowing or Drainage Issues

- Abrasive Collection: Check the two abrasive collection buckets. If they are full of used garnet, water cannot drain properly and will overflow the tank.

- Drain Filter: Inspect the internal filters and drain screens for debris or heavy sediment buildup.

- Pump Cycles: If the tank isn't draining, ensure the internal circulation pump is functioning correctly.

When in Doubt

- STOP the machine: If the water jet cuts through the bed or sprays erratically, hit the E-Stop immediately.

- Dry the Area: The Wazer uses high-pressure water; if leaks occur, power down the machine before touching any electrical components.

- Abrasive Disposal: Never pour used garnet down a standard sink drain; it will cause a permanent clog.

- Ask for help: Waterjets are complex; if the pump sounds like it is "cavitating" (loud, irregular popping), consult a supervisor.

Quick Reference Guide: Formlabs Form 3B (SLA Printer)

SAFETY FIRST (READ BEFORE USE)

- Chemical Safety: Resin is a skin sensitizer. Always wear nitrile gloves and safety glasses when handling resin tanks, build platforms, or uncured prints.

- Ventilation: Operate in a well-ventilated area to minimize exposure to resin vapors.

- UV Exposure: Keep the printer cover closed during operation to prevent accidental UV curing of resin in the tank.

- Spill Response: If resin is spilled, clean immediately with Isopropyl Alcohol (IPA) and paper towels.

BASIC OPERATION

Power-On Sequence:

- Plug in the printer and wait for the touchscreen to initialize.

- Ensure the resin tank is properly seated and the mixer is attached.

- Insert a resin cartridge and open the vent cap.

Printing:

- Prepare the file in PreForm software, selecting the correct material and layer thickness.

- Send the job to the printer via Wi-Fi, Ethernet, or USB.

- On the printer's touchscreen, select the job and press Print.

- Confirm the build platform and resin tank match the file requirements.

POST-PROCESSING

- Remove the build platform and place it in the Form Wash (IPA bath) for the specified time.

- Once dry, remove supports carefully using snips.

- Place the part in the Form Cure to reach final mechanical properties.

TROUBLESHOOTING

Printer Will Not Start or Screen is Blank

- Check Power Connection: Ensure the power cable is firmly seated in the back of the printer and the outlet.

- Power Cycle: Turn the printer off using the switch at the back, wait 30 seconds, and turn it back on.

- Firmware Hang: If the screen is frozen, unplug the machine for one minute and restart. Ensure the printer is connected to Wi-Fi or Ethernet to check for mandatory firmware updates.

Print Fails to Adhere to Build Platform

- Clean the Platform: Ensure the aluminum build surface is completely free of cured resin or oil. Clean with 90% Isopropyl Alcohol (IPA).

- Check Resin Level: Verify the cartridge is not empty and the bite valve is not stuck shut, preventing resin from filling the tank.

- Tank Inspection: Check the bottom of the resin tank (the film) for "ghosting" or cured debris. Debris on the film will block the LFS laser and prevent the first layer from forming.

? Resin Not Filling Tank

- Bite Valve Check: Inspect the rubber bite valve at the bottom of the resin cartridge. If it is stuck, gently squeeze it with gloved fingers to loosen the seal.

- Air Vent: Ensure the vent cap on top of the resin cartridge is OPEN during printing to allow for proper flow.

- Cartridge Chip: Ensure the ID chip on the bottom of the cartridge is clean and making contact with the printer's reader.

"Missing Tank" or "Missing Cartridge" Error

- Clean Contacts: Use a cotton swab dampened with IPA to gently clean the small gold spring contacts on the printer and the corresponding chips on the tank/cartridge.

- Re-seat Hardware: Remove the tank or cartridge and slide it back in firmly until it clicks or the printer acknowledges it.

Debris in Resin Tank (Cured Flakes)

- Filter the Resin: If a print fails, there may be small bits of cured plastic floating in the liquid. Use a fine mesh paint filter to strain the resin before starting a new job.

- Cleaning Mesh: Use the "Cleaning Mesh" feature on the printer's maintenance menu to create a thin cured sheet that pulls all debris off the bottom of the tank at once.

When in Doubt

- Wear Gloves: Always wear nitrile gloves when handling resin or internal components.

- Protect the Glass: Never touch the optical window (the glass under the tank) with bare fingers. If it is dirty, use only PEC*PADs and 99% IPA.

- No Scrapers: Never use a metal scraper on the flexible tank film; use only the provided plastic tank tool.

- Ask for help: SLA printing involves hazardous chemicals; if a spill occurs inside the printer, notify a supervisor immediately.

Quick Reference Guide: ShopBot CNC Milling Table

SAFETY FIRST (READ BEFORE USE)

- Constant Supervision: NEVER leave the CNC unattended while it is running.

- Personal Safety: Tie back long hair, remove loose clothing, and avoid dangling jewelry to prevent entanglement with moving axes or the spindle.

- Emergency Stops: Familiarize yourself with the location of the Emergency Stop button before starting any job.

- Eye and Ear Protection: Always wear safety glasses and hearing protection when the machine is in operation.

- Debris Management: Ensure the work area is clear of tools and debris that could interfere with the gantry movement.

BASIC OPERATION

Power-On Sequence:

- Turn ON the main power switch on the control box.

- Launch the ShopBot control software on the connected PC.

- Ensure the Emergency Stop is released.

- Perform the Reset routine to engage the stepper motors.

- Run the Spindle Warm-up routine if the machine has been idle.

Zeroing and Files:

- Secure your material firmly to the sacrificial bed using clamps or screws.

- Use the Z-Zero Plate to automatically set the bit height above the material.

- Manually jog the machine to the starting corner of your material and set the X and Y Zero.

- Load your .sbp or .gcode file into the control software.

- Perform a "3D Offset" air-cut or frame the job to ensure the toolpath stays within the material bounds.

TROUBLESHOOTING

- Machine Has Power but Won't Move: Check if the E-Stop is engaged or if the control panel needs to be restarted.

- Poor Cut Quality: Check for a dull bit, incorrect spindle speed, or loose material clamping.

- Communication Lost: Power cycle the entire machine and wait 30 seconds before restarting the software.

Quick Reference Guide: Rabbit Laser USA RL-80-1290

SAFETY FIRST (READ BEFORE USE)

- Supervision: NEVER leave the laser unattended while running.

- Observation: Always remain in the room and watch the job from start to finish.

- Personal Safety: Tie back long hair, remove loose clothing, and avoid dangling jewelry.

- Safety Features: Do not bypass or disable safety features.

- Fire Preparedness: Laser cutting can ignite materials; be prepared at all times.

- Materials NOT Allowed: Never cut or engrave PVC/Vinyl, Polycarbonate, ABS, Fiberglass, or unknown foams.

BASIC OPERATION

Power-On Sequence:

- Turn ON water chiller.

- Turn ON exhaust fan.

- Turn ON air assist.

- Turn ON main laser power.

- Power on the Ruida control panel and wait for system stabilization.

Focal Length Setup:

- Place material flat on the laser bed.

- Move laser head over material.

- On control panel, select Auto Focus / Auto Adjust.

- If manual focus is used, place focus gauge between nozzle and material and raise bed until it fits snugly.

TROUBLESHOOTING

- Laser Will Not Fire: Check that the lid is fully closed, the water chiller is running, and the emergency stop is released.

- Cuts Not Going Through: Check focus, clean lens and mirrors, or increase power/slow speed.

- Water Flow Alarm: Check water level and ensure hoses are not kinked; do NOT bypass the alarm.

- Smoke Not Clearing: Verify the exhaust fan is ON and ducting is unobstructed

Quick Reference Guide: Original Prusa i3 MK3S+

SAFETY FIRST (READ BEFORE USE)

- Heat Hazard: The nozzle can reach temperatures up to 280°C and the heatbed up to 120°C; do not touch during or immediately after operation.

- Moving Parts: Keep hands, hair, and loose clothing away from the X, Y, and Z-axis motors and belts while the printer is active.

- Pinch Points: Avoid reaching into the frame while the printer is homing or printing.

- Ventilation: Ensure the printer is in a well-ventilated area, especially when printing materials like ABS or ASA.

BASIC OPERATION

Power-On Sequence:

- Check that the removable magnetic steel sheet is clean and properly aligned on the heatbed.

- Switch the power button (located on the side of the power supply) to ON.

- Wait for the LCD screen to display the main menu.

Loading Filament:

- Select Preheat on the LCD menu and choose the material type (e.g., PLA, PETG).

- Once the nozzle reaches the target temperature, select Load Filament.

- Insert the filament into the extruder intake and follow the on-screen prompts until color flows from the nozzle.

Starting a Print:

- Insert the SD card containing your .gcode file (prepared in PrusaSlicer).

- Select Print from SD on the menu.

- Choose your file and press the control knob to begin.

- Watch the First Layer Calibration to ensure the plastic adheres properly to the bed.

TROUBLESHOOTING

- Poor Adhesion: Clean the steel sheet with 90%+ Isopropyl Alcohol to remove oils; avoid touching the print surface with bare fingers.

- First Layer Issues: If the nozzle is too high or too low, use the Live Adjust Z setting during the first layer to fine-tune the height.

- Thermal Error: Check that the room is not too cold and that the printer is away from direct drafts.

Quick Reference Guide: Mole V2.2 3D Scanner

SAFETY FIRST (READ BEFORE USE)

- Eye Safety: Avoid looking directly into the scanner’s light source for extended periods during operation.

- Cable Management: Ensure the connection cables are positioned to prevent tripping or accidental pulling of the scanner or connected PC.

- Handling: The scanner is a precision optical instrument; avoid dropping or subjecting it to heavy vibrations.

- Environmental Conditions: Operate in a dry environment away from direct, harsh sunlight which can interfere with the infrared sensors.

BASIC OPERATION

Setup:

- Connect the Mole V2.2 to your computer using the provided USB and power cables.

- Launch the JMStudio software.

- Ensure the scanner is recognized by the software (indicated by a live preview window).

Scanning Modes:

- Turntable Mode:

  - Place the object on the center of the turntable.

  - Select Turntable Mode in JMStudio.

  - Click Initial to scan the background, then place the object and click Scan.

- Handheld Mode:

  - Select Handheld Mode in the software.

  - Maintain a distance of 15cm to 50cm from the object.

  - Slowly move the scanner around the subject, ensuring the "distance indicator" stays in the green zone.

Post-Processing:

- Once the scan is complete, click Stop.

- Use the Process button to align multiple scans and generate a point cloud.

- Export the final model as an .STL or .OBJ for 3D printing or CAD use.

TROUBLESHOOTING

- Tracking Lost: If the software loses the object's position, move the scanner back to a previously scanned area with distinct features to regain tracking.

- Difficult Surfaces: For dark, shiny, or transparent objects, apply a light coat of 3D scanning spray or baby powder to create a matte surface.

- Alignment Issues: Use "Manual Alignment" in the software if the automatic process fails to merge two scan rotations correctly.

Quick Reference Guide: JET 12" Drill Press

SAFETY FIRST (READ BEFORE USE)

- Eye Protection: Always wear safety glasses or a face shield when operating the drill press.

- Entanglement Hazard: Tie back long hair, remove loose clothing, and avoid dangling jewelry.

- No Gloves: Do not wear gloves while operating the spindle, as they can be caught in the rotating drill bit.

- Secure Workpiece: Always use a drill press vise or clamps to secure your material; never hold a workpiece by hand while drilling.

- Chuck Key: Never leave the chuck key in the chuck; remove it immediately after tightening or loosening a bit.

BASIC OPERATION

Power-On Sequence:

- Adjust the table height and lock it in place.

- Insert the drill bit into the chuck and tighten it securely with the chuck key.

- Ensure the belt speed is set correctly for the material and bit size (refer to the speed chart inside the belt cover).

- Switch the power to ON.

Drilling:

- Mark your hole location with a center punch to prevent bit "walk."

- Slowly lower the spindle using the feed handles until the bit contacts the material.

- Apply consistent pressure, backing the bit out frequently (peck drilling) to clear chips and prevent overheating.

- Turn the machine OFF and wait for the spindle to stop completely before removing the workpiece.

TROUBLESHOOTING

- Machine Will Not Start: Check that the plug is secure and the emergency stop (if equipped) is released.

- Bit Slipping: Ensure the chuck is tightened properly using all three holes on the chuck body.

- Excessive Noise/Vibration: Check that the belt tension is correct and the bit is not bent or dull.

- Smoke or Burning: Reduce spindle speed, check focus on the cut, or use a sharper bit.

Quick Reference Guide: WEN 16" Scroll Saw

SAFETY FIRST (READ BEFORE USE)

- Eye Protection: Always wear safety glasses or a face shield when operating the saw.

- Fingers Clear: Keep your fingers to the side of the blade path, never directly in front of it.

- Tension Check: Ensure the blade is properly tensioned before turning the machine on; a loose blade can snap or wander.

- Hold-Down Foot: Adjust the hold-down foot so it rests lightly on top of your workpiece to prevent the wood from lifting.

BASIC OPERATION

Power-On Sequence:

- Check that the blade teeth are pointing downward.

- Adjust the air blower nozzle to clear sawdust from your cut line.

- Turn the speed dial to the lowest setting before flipping the power switch ON.

- Gradually increase speed based on the thickness and hardness of the material.

Blade Changes:

- Turn OFF and unplug the saw.

- Release the tension lever.

- Loosen the thumb screws on the upper and lower blade holders to swap the blade.

TROUBLESHOOTING

Machine Will Not Start

- Check Power Switch: Ensure the power switch is in the ON position and the unit is plugged into a functional 120V outlet.

- Variable Speed Dial: If the dial is set to the absolute minimum, some motors may stall. Turn the dial to a medium setting and try again.

- Safety Key: Many WEN models have a removable yellow plastic safety key in the power switch. If it is missing, the saw will not engage.

Blade Keeps Breaking

- Over-Tensioning: If the blade snaps immediately upon starting, the tension lever is likely set too tight. It should "ping" like a guitar string, not feel brittle.

- Aggressive Feeding: Do not force the wood into the blade. If you have to push hard, the blade is either dull or you are moving too fast for the material thickness.

- Blade Orientation: Ensure the teeth are pointing DOWN toward the table. If they point up, the workpiece will chatter violently and snap the blade.

Excessive Vibration or Workpiece "Jumping"

- Hold-Down Foot: Adjust the metal hold-down foot so it rests lightly on top of your wood. If there is a gap, the wood will chatter against the table.

- Table Bolt: Check the locking knob under the table. If the table is not locked at 0° (or your desired angle), it will vibrate and create inaccurate cuts.

- Mounting: Ensure the saw is bolted to a sturdy workbench or stand. Benchtop scroll saws are prone to "walking" if not secured.

Air Blower Not Clearing Dust

- Tube Alignment: Ensure the flexible plastic air tube is pointed directly at the contact point between the blade and the wood.

- Kinked Hose: Check the internal bellows and the clear tubing for kinks or disconnects that prevent air from reaching the nozzle.

Blade Slipping Out of Clamps

- Clean the Clamps: Sawdust or oil on the blade ends can cause them to slip. Wipe the blade ends and the inside of the clamps with a dry cloth.

- Tighten Thumb Screws: If using pinless blades, ensure the setscrews are tightened firmly with the provided hex key or thumb knob.

When in Doubt

- STOP the machine: If the blade breaks, turn off the power before reaching in to retrieve the broken pieces.

- Unplug for Maintenance: Always unplug the saw before changing blades or adjusting internal components.

- Check Blade Type: Ensure you are using the correct blade (Pinned vs. Pinless) for your specific holder setup.

- Ask for help: If the motor hums but the arm doesn't move, the internal linkage may be jammed; consult a supervisor.

Quick Reference Guide: Grizzly 17" 2 HP Bandsaw

SAFETY FIRST (READ BEFORE USE)

- Blade Guard Height: Adjust the upper blade guide to be no more than 1/4" above the material.

- Entanglement: Tie back long hair and avoid loose clothing; do not wear gloves.

- Push Sticks: Use a push stick for any cuts where your fingers would come within 3 inches of the blade.

- Wait for Stop: Never back a piece of wood out of a cut while the blade is moving, as it can pull the blade off the wheels.

BASIC OPERATION

- Open the wheel covers and check blade tracking/tension (ensure the saw is unplugged).

- Turn on the dust collection system.

- Flip the switch to ON and allow the blade to reach full speed.

- Feed the material slowly, allowing the blade to cut without forcing it.

TROUBLESHOOTING

Machine Has Power but Won't Move

- Check E-Stop: Verify that the red Emergency Stop button is not depressed.

- Door Safety Switches: Ensure both the upper and lower wheel covers are completely closed and latched. This saw features interlock switches that prevent the motor from starting if the doors are ajar.

- Foot Brake: Ensure the foot brake is fully released. If the brake is engaged, the internal safety switch will disconnect power to the motor.

- Power Reset: Turn the main power switch to OFF, wait 30 seconds, and turn it back ON to reset the magnetic starter.

Blade Wandering or Inaccurate Cuts

- Check Tension: Verify the blade is tensioned correctly for its width using the scale on the upper wheel. A loose blade will "bow" and wander during thick cuts.

- Guide Bearings: Ensure the upper and lower guide bearings are adjusted to roughly 0.004" (the thickness of a piece of paper) from the blade.

- Blade Guard Height: Confirm the guard is set no more than 1/4" above the material. Excessively high guards allow the blade to twist.

- Dull Blade: If the blade requires excessive force to cut or pulls to one side despite proper setup, the teeth are likely dull. Replace the blade.

Excessive Vibration or Noise

- Blade Tracking: Turn the machine OFF and rotate the wheels by hand to ensure the blade is centered on the tires.

- Wheel Tires: Check for built-up sawdust or "pitch" on the rubber tires. Clean them with a stiff brush; debris causes the blade to "bounce."

- Drive Belt: Inspect the V-belt for cracks, slipping, or debris.

Motor Bogs Down or Smokes

- Feed Rate: Slow down. Let the speed of the blade do the work without forcing the wood.

- Dust Collection: Ensure the dust collector is ON. Packed sawdust in the lower cabinet creates friction against the lower wheel and can cause the motor to overheat.

- Incorrect TPI: Ensure you are using the correct Teeth Per Inch (TPI) for your material. Fewer teeth (3-4 TPI) are required for thick resawing to prevent heat buildup.

When in Doubt

- STOP the machine: If you hear a loud "bang" (likely a snapped blade), immediately step back and hit the E-Stop or foot brake.

- Power down safely: Unplug the machine before opening wheel covers or performing any blade changes.

- Ask a supervisor: Bandsaws require precise "tuning." If you are unsure about tracking or tension, ask for a trained operator's assistance.

- Never "test" fixes: Do not attempt to adjust guide bearings while the blade is in motion.

Quick Reference Guide: DeWALT 13" Planer

SAFETY FIRST (READ BEFORE USE)

- Ear Protection: This machine is extremely loud; hearing protection is mandatory.

- Nails/Hardware: Inspect all wood for nails, staples, or dirt before feeding; hitting metal will ruin the blades instantly.

- No Reaching In: Never reach into the planer while it is running.

- Material Length: Do not plane boards shorter than 12 inches to avoid them getting stuck or kicked back.

BASIC OPERATION

- Measure the thickest part of your board.

- Set the planer height to roughly 1/32" less than the board thickness.

- Turn on the dust extractor (required to prevent chip buildup).

- Start the motor and feed the board into the front of the machine; let the rollers pull it through.

TROUBLESHOOTING

Machine Will Not Start

- Check the Circuit Breaker: The planer has an onboard circuit breaker (usually a small button near the power switch). If the machine was overloaded, press this button to reset it.

- Power Switch Lock: Ensure the yellow safety key is fully inserted into the paddle switch.

- Overheating: If the motor shuts off during heavy use, allow it to cool for 15–20 minutes before attempting to restart.

Boards Are Not Feeding (Getting Stuck)

- Clean the Tables: Sap and dust buildup on the infeed and outfeed tables create friction. Clean the tables with a degreaser and apply a thin coat of paste wax.

- Clean the Rollers: Unplug the machine and wipe the rubber feed rollers with a cloth dampened with denatured alcohol to restore their "grip."

- Depth of Cut: You may be trying to remove too much material. Reduce the depth of cut (aim for 1/32" or less for wide or hard boards).

- Table Alignment: Ensure the extension tables are level with the main granite/steel bed. If they sag, the board will "nose dive" and stop feeding.

Poor Surface Quality (Snipe or Ridges)

- Snipe (Gouge at ends): This is caused by the board tilting as it enters or leaves the machine. Lift the far end of the board slightly as it enters, or use "sacrificial" scrap wood before and after your workpiece.

- Ridges/Lines: This indicates a nick in the knives (usually from hitting a staple or sand). The DeWALT 735 has two-sided knives; you can shift one knife slightly to the left or right to offset the nicks, or flip/replace them.

- Tear-out: If the wood grain is fuzzy or chipped, try feeding the board in the opposite direction or reduce the feed speed (if using the 2-speed Model 735).

Excessive Dust or Chips Blowing Out the Front

- Dust Collection: This machine has an internal fan that forces chips out. If you are not connected to a vacuum or dust collector, the chips will clog the cutter head and blow back at you.

- Clogged Chute: Unplug the machine and remove the dust shroud to check for "chip packing," which happens if the vacuum is too weak or the wood is very wet.

When in Doubt

- STOP the machine: If you hear a high-pitched scream or the motor "bogs down" significantly, hit the stop paddle immediately.

- Unplug for Maintenance: Never reach near the cutter head or change knives without physically disconnecting the power cord.

- Check Material: Never plane wood that has paint, glue, or embedded metal.

- Ask for help: Changing the three knives on a DeWALT planer requires precision; if the cuts are uneven, consult a supervisor for blade replacement

Quick Reference Guide: Baileigh Metal Horizontal Bandsaw BS-210H

SAFETY FIRST (READ BEFORE USE)

- Coolant System: Ensure the coolant pump is ON and flowing to prevent the blade from overheating and snapping.

- Vise Security: Always clamp the metal workpiece tightly in the vise; never hold metal by hand.

- Blade Guard: Keep the adjustable blade guide as close to the workpiece as possible for maximum support.

BASIC OPERATION

- Open the vise, insert material, and tighten the handwheel.

- Set the descent rate (hydraulic feed) so the saw drops slowly into the metal.

- Turn the power ON.

- The saw will automatically shut off once the cut is complete.

TROUBLESHOOTING

Machine Will Not Start

- Check E-Stop: Ensure the mushroom-style Emergency Stop button is twisted and released.

- Trigger Switch: For manual operation, the trigger switch on the handle must be fully depressed. For hydraulic descent, ensure the selector switch is set to the correct mode.

- Limit Switch: The saw features an automatic shut-off switch that triggers when the arm reaches the bottom of its travel. If the arm is already down, the motor will not start.

- Blade Tension Switch: Many Baileigh models have a safety sensor that prevents the motor from running if the blade is too loose. Check the tension gauge.

Blade Is Stalling or Jumping

- Check Tension: If the blade "slips" on the drive wheels under load, increase the tension using the handwheel until the gauge reaches the green zone.

- Speed Selection: You may be using a speed that is too high for the material. Shift to a lower gear/speed setting for harder metals like stainless steel.

- Drive Belt: Open the motor access panel (unplugged) to ensure the V-belt is tight and not glazed or slipping.

Inaccurate or Crooked Cuts

- Blade Guides: Ensure the adjustable sliding blade guide is moved as close to the workpiece as possible. This minimizes blade twist and deflection.

- Guide Bearings: Inspect the ceramic or roller guides. They should be snug against the blade without binding it. If there is a gap, the blade will wander.

- Feed Rate: If the hydraulic descent is set too fast, the blade will "lead" or curve. Turn the hydraulic flow control valve clockwise to slow the descent.

Excessive Heat or Blade Wear

- Coolant System: Verify the coolant pump is running and the nozzle is aimed at the entry point of the cut. Check the reservoir for metal "chips" blocking the pump intake.

- TPI (Teeth Per Inch): Ensure you are following the "3-Tooth Rule"—at least three teeth must be in contact with the material at all times. If the teeth are too large for thin-walled tubing, they will strip off.

When in Doubt

- STOP the machine: If the blade snaps or the material slips out of the vise, hit the E-Stop immediately.

- Secure the Vise: Never attempt to "hold" metal by hand. If the piece moves, it will snap the blade and potentially cause injury.

- Hydraulic Lock: Always lock the hydraulic cylinder before changing blades to prevent the heavy saw arm from falling unexpectedly.

- Ask for help: Metal bandsaws use high-tension blades and heavy castings; if the descent is erratic, consult a supervisor for hydraulic adjustment.

Quick Reference Guide: Trinco Sandblaster Model 48/BP2

SAFETY FIRST (READ BEFORE USE)

- Seal Check: Ensure the door latches are secure before activating the air blast to prevent dust from entering the room.

- Window Guard: Do not operate if the viewing window is too frosted to see the workpiece clearly.

- Gloves: Use only the integrated heavy-duty rubber gloves provided with the cabinet.

BASIC OPERATION

- Place the part inside the cabinet and close the door securely.

- Turn on the internal light and the dust collector/vacuum.

- Insert your arms into the gloves and grasp the blast nozzle and the part.

- Depress the foot pedal to start the flow of abrasive; move the nozzle in a steady motion across the part.

TROUBLESHOOTING

Low Blasting Pressure / Weak Stream

- Check Air Supply: Ensure the shop compressor is on and the regulator is set to the correct PSI (typically 60-100 PSI).

- Clogged Nozzle: Unplug the air and inspect the ceramic nozzle. Debris or oversized media can block the tip.

- Foot Pedal: Ensure the foot pedal is fully depressed and the air lines leading to it are not kinked or leaking.

- Moisture in Air: If the stream is "pulsing" or "spitting," there is water in the lines. Drain the water trap/filter located on the back of the machine.

Media Not Flowing (Air Only, No Abrasive)

- Check Media Level: Ensure there is enough abrasive in the hopper. The suction tube must be submerged in the media.

- Damp Media: If the abrasive has absorbed moisture, it will "bridge" in the hopper and won't flow. Replace with dry media or sift the existing batch.

- Suction Tube Blockage: Inspect the pickup tube at the bottom of the hopper for clogs or large debris (like paint chips or rust flakes).

- Mixing Valve: Adjust the air/media mixing valve at the base of the hopper to ensure the ratio is correct.

Poor Visibility Inside Cabinet

- Dust Collector: Ensure the BP2 dust collector (vacuum) is ON. If it is running but the cabinet is still cloudy, the filter bag or cartridge is likely full and needs to be shaken out or replaced.

- Worn Window Shield: The plastic "under-sheet" of the viewing window becomes frosted over time. If you cannot see the workpiece, replace the sacrificial plastic film.

- Internal Light: Check the fluorescent or LED bulb inside the cabinet. If it’s out, the media may have damaged the protective cover.

Media Leaking from Cabinet

- Door Seals: Inspect the rubber gaskets around the doors. If they are torn or brittle, dust will escape.

- Glove Holes: Check the integrated gloves for tears. Even a small hole will spray high-pressure dust onto the operator.

- Negative Pressure: Ensure the dust collector is pulling enough air to maintain negative pressure inside the cabinet, which keeps dust from leaking out of small gaps.

When in Doubt

- STOP the machine: Release the foot pedal immediately if a glove fails or the window breaks.

- Static Shock: Sandblasting creates significant static electricity. Ensure the machine is properly grounded to avoid shocks.

- Never Open While Blasting: Wait for the dust to settle and the vacuum to clear the air before opening the side doors.

- Ask for help: If the blast stream is inconsistent despite a full hopper, the internal abrasive hose may be worn through and require replacement by a supervisor.

Quick Reference Guide: Universal Robots UR10e / UR5e

SAFETY FIRST (READ BEFORE USE)

- Collaborative Safety: While designed to work alongside humans, always perform a risk assessment for the specific end-effector (gripper/tool) being used.

- Emergency Stop: Ensure the E-Stop on the Teach Pendant is within reach at all times during manual or automatic operation.

- Pinch Points: Keep hands clear of joints and the mounting flange during movement, especially when the robot is in "Free Drive" mode.

- Backdrive: In the event of an emergency or power loss, the joints can be moved manually by applying significant force; use this only for safety extraction.

BASIC OPERATION

Power-On Sequence:

- Turn the power switch on the Control Box to ON.

- On the Teach Pendant, tap the Power Icon (bottom left) and select Power On.

- Once initialized, tap Release Brakes. You will hear a series of clicks as the joints unlock.

- The robot status light on the top of the pendant will turn green when ready.

Programming & Movement:

- Free Drive: Hold the black button on the back of the Teach Pendant to manually move the robot arm to a desired position.

- Waypoints: Use the PolyScope interface to "Add Waypoint" and save positions for your program.

- Graphics Tab: Use the 3D visualization to verify the robot's intended path before running the program at full speed.

- Speed Slider: Always start a new program with the speed slider set to 10% or lower to check for collisions.

TROUBLESHOOTING

- Protective Stop: This occurs if the robot detects a collision or abnormal force. Check for obstructions, then tap the popup to Resume.

- Joint Limit: If a joint reaches its mechanical limit, use the jog arrows or Free Drive to move the robot back into its working envelope.

- Communication Error: Check that the cable between the Control Box and the Robot Arm is securely connected and the locking ring is tightened.

Quick Reference Guide: Haas Minimill

SAFETY FIRST (READ BEFORE USE)

- Enclosure Safety: Never bypass or disable the door safety interlocks.

- Eye Protection: Always wear safety glasses when the machine is powered on to protect against flying chips and coolant splashes.

- Tooling Safety: Ensure all tools are securely seated in the tool changer and that pull studs are tightened to the correct specification.

- Clearance Check: Always "dry run" new programs with the Rapid Override turned down to check for potential tool crashes or clearance issues.

- Entanglement Hazard: Tie back long hair, remove loose clothing, and avoid dangling jewelry; do not wear gloves while the spindle is in motion.

BASIC OPERATION

Power-On Sequence:

- Turn the main breaker (located on the rear of the machine) to the ON position.

- Press the Power Up/Restart button on the control pendant.

- The machine will automatically home all axes (X, Y, and Z).

- Ensure the air pressure is at the required level (typically 85 psi) before operating.

Setting Offsets:

- Work Offset: Use the jog handle and an edge finder or probe to locate the part's X and Y coordinates; input these into the G54 table.

- Tool Offset: Lower each tool to the top of the workpiece or a tool setter to define the Z-axis length; save these in the Tool Offset table.

Running a Job:

- Upload your program via USB or network connection.

- Verify the program using the Graphics mode on the screen.

- Select Memory mode, turn on the coolant system, and press Cycle Start.

TROUBLESHOOTING

- Machine Will Not Move: Check if the Emergency Stop is pressed or if the door is open.

- Alarm Active: Read the alarm code on the screen and refer to the Haas manual or press the Help button for a description.

- Tool Change Error: Ensure there is no debris in the tool changer carousel and that the air pressure is sufficient.

Quick Reference Guide: PEXTO PX36-A (Box and Pan Brake) & 137-L (Foot Shear)

SAFETY FIRST (READ BEFORE USE)

- PPE Mandatory: Always wear safety glasses and heavy-duty work gloves when handling sheet metal to prevent lacerations from sharp edges.

- Capacity Limits: Never exceed the rated material thickness (typically 16 gauge mild steel); attempting to shear or bend thicker material can damage the blades or frame.

- Pinch Points: Keep fingers clear of the clamping bar and the cutting blade at all times.

- Foot Safety: Ensure your feet are clear of the foot treadle area before others operate the machine, and wear closed-toed shoes.

BASIC OPERATION: PEXTO 137-L (Foot Shear)

- Alignment: Place the sheet metal on the bed and align it against the side gauge or back gauge for a square cut.

- Clamping: Ensure the material is flat against the bed to prevent "creeping" during the cut.

- Shearing: Place one foot firmly on the treadle and apply a quick, decisive downward force to shear the metal.

- Retrieval: Carefully remove the off-cut from the rear of the machine, being mindful of sharp burrs.

BASIC OPERATION: PEXTO PX36-A (Box and Pan Brake)

- Finger Setup: For box folding, remove or rearrange the upper fingers to match the width of your workpiece.

- Clamping: Slide the metal under the fingers to your bend line and pull the clamping handle forward to lock the material in place.

- Bending: Lift the folding beam handles slowly to the desired angle, using the stop gauge for repeatable bends.

- Release: Push the clamping handle back to release the fingers and remove the part.

TROUBLESHOOTING

- Inaccurate Bends: Check that all fingers are tightened securely and that the material is not slipping during clamping.

- Heavy Burrs on Shear: Inspect the blade for nicks or dullness; ensure the blade gap is adjusted correctly for the material thickness.

- Machine Sticking: Lightly lubricate moving joints and the blade faces with machine oil.

Quick Reference Guide: Clausing Lathe

SAFETY FIRST (READ BEFORE USE)

- No Gloves: Never wear gloves while operating the lathe, as they can be easily caught in the rotating chuck or workpiece.

- Entanglement Hazard: Tie back long hair, remove loose clothing, and avoid dangling jewelry.

- Eye Protection: Always wear safety glasses or a face shield; flying metal chips are hot and sharp.

- Chuck Key: Never leave the chuck key in the chuck; remove it immediately after tightening or loosening.

- Clearance: Ensure the tool post and carriage have sufficient clearance before starting the spindle to prevent a crash.

BASIC OPERATION

Power-On Sequence:

- Check that the spindle speed is set correctly for the material and diameter being turned.

- Ensure the gear levers are fully engaged and not in a neutral or "between" position.

- Ensure the Emergency Stop is released.

- Switch the main power to ON and engage the spindle lever.

Operational Steps:

- Mounting: Secure the workpiece firmly in the chuck or between centers.

- Tool Setup: Set the cutting tool height exactly to the center line of the spindle.

- Manual Jogging: Use the handwheels to move the carriage and cross-slide to the starting position.

- Cutting: Engage the power feed only after confirming the feed rate and direction on the quick-change gearbox.

TROUBLESHOOTING

- Machine Will Not Start: Verify the foot brake is not engaged and the chuck guard (if equipped) is fully closed.

- Excessive Chatter: Reduce the spindle speed, increase the feed rate, or check that the tool overhang is minimized.

- Poor Surface Finish: Check for a dull cutting edge or ensure the tool is positioned exactly at center height.

Quick Reference Guide: Miller Welding Machine

SAFETY FIRST (READ BEFORE USE)

- PPE Mandatory: Always wear a welding helmet (correct shade), flame-resistant gloves, and leather boots.

- Ventilation: Ensure the exhaust fan is ON; never weld in a confined space without proper airflow.

- Fire Safety: Clear all flammable materials, including rags and solvents, from the welding area before starting.

- Dry Conditions: Never weld in damp or wet conditions; ensure the ground clamp is attached directly to clean metal.

BASIC OPERATION

Power-On Sequence:

- Connect the ground clamp to the workpiece or the welding table.

- Open the shielding gas cylinder valve slowly and check the regulator pressure.

- Switch the main power toggle to ON.

- Adjust the voltage and wire feed speed (MIG) or amperage (TIG/Stick) based on the material thickness.

Shut-Down Sequence:

- Turn OFF the power switch.

- Close the gas cylinder valve completely.

- Trigger the torch to bleed the remaining gas from the lines until the regulator gauge reads zero.

TROUBLESHOOTING

- Porous Welds: Check if the gas cylinder is empty or if there is a draft blowing the shielding gas away from the puddle.

- Wire Stops Feeding: Check for a bird's nest at the drive rolls or a clogged contact tip.

- Weak Arc: Ensure the ground clamp is attached to a clean, unpainted surface.

Quick Reference Guide: Hobart IronMan 230 Welding Machine

SAFETY FIRST (READ BEFORE USE)

- PPE Mandatory: Always wear a welding helmet with the correct shade, flame-resistant gloves, and a leather apron.

- Ventilation: Ensure the exhaust system is ON or work in a well-ventilated area to avoid inhaling toxic fumes.

- Fire Hazard: Clear the area of all flammable materials (rags, paper, solvents) before striking an arc.

- Electrical Safety: Never weld in damp or wet conditions. Ensure the machine is properly grounded.

BASIC OPERATION

Power-On Sequence:

- Connect the ground clamp to the workpiece or welding table.

- Open the valve on the shielding gas cylinder slowly.

- Switch the main power toggle to ON.

- Adjust voltage and wire feed speed based on the material thickness chart found inside the side panel.

Shut-Down Sequence:

- Turn OFF the welding machine power.

- Close the shielding gas cylinder valve completely.

- Trigger the gun to bleed the remaining gas from the lines until the regulator reads zero.

TROUBLESHOOTING

- Porous Welds: Check if the gas cylinder is empty or if there is a draft blowing away the shielding gas.

- Wire Nesting (Bird Nest): Stop immediately. Check the drive roll tension and ensure the contact tip is not clogged.

- Weak Arc: Verify the ground clamp is attached to clean, unpainted metal.

Quick Reference Guide: Geo Knight Cap Heat Press / USCutter Digital Heat Press

SAFETY FIRST (READ BEFORE USE)

- Heat Hazard: The heating platen can reach temperatures exceeding 200°C; avoid all skin contact with the heated surfaces to prevent severe burns.

- Pinch Hazard: Keep hands and fingers clear of the hinge and clamping mechanism when closing the press.

- Fumes: Ensure adequate ventilation when pressing synthetic fabrics or vinyl that may release odors when heated.

- Electrical: Do not operate with a damaged power cord or in damp environments.

BASIC OPERATION

Power-On Sequence:

- Plug the unit into a grounded outlet and switch the power to ON.

- Set the desired Temperature and Time using the digital control panel.

- Wait for the machine to reach the target temperature before use.

Pressing Procedure:

- Position the garment or cap on the lower platen/form.

- Place the transfer or vinyl accurately on the material.

- Lower the handle and lock the press into place.

- When the timer sounds, carefully lift the handle and remove the material.

TROUBLESHOOTING

- Inconsistent Transfers: Check that the pressure adjustment knob is set correctly for the thickness of the material.

- Machine Not Heating: Ensure the emergency stop (if equipped) is not engaged and the fuse/breaker has not tripped.

- Material Scorching: Reduce the temperature or the pressing time.

Quick Reference Guide: Bucktool Belt and Disc Sander

SAFETY FIRST (READ BEFORE USE)

- Eye Protection: Always wear safety glasses or a face shield; the sander generates high-speed dust and debris.

- Entanglement: Do not wear gloves, loose clothing, or jewelry; tie back long hair.

- Grip: Never hold small workpieces by hand; use a miter gauge or a jig to maintain control.

- Work Direction: Always sand on the downward-moving side of the disc to prevent the workpiece from being lifted and thrown.

BASIC OPERATION

- Check that the belt and disc are in good condition and not torn or excessively clogged.

- Adjust the work table to the desired angle and tighten the locking knobs.

- Turn ON the dust collection system.

- Switch the sander ON and allow it to reach full speed.

- Hold the workpiece firmly against the table and feed it lightly against the abrasive surface.

TROUBLESHOOTING

- Belt Tracking: If the belt moves toward one side, use the tracking adjustment knob to center it while the machine is running.

- Excessive Heat: Do not force the material; use lighter pressure and allow the abrasive to do the work to prevent burning the wood or clogging the paper.

Quick Reference Guide: Legacy Lulzbot TAZ 6

SAFETY FIRST (READ BEFORE USE)

- Heat Hazard: The hot end reaches 300°C and the bed reaches 120°C; do not touch until cooled.

- Moving Parts: Keep the print area clear of tools and hands while the gantry is moving.

- Automatic Cleaning: The printer performs a nozzle-wipe routine before every print; keep hands clear of the wiping pad area.

BASIC OPERATION

- Turn on the power switch on the side of the control box.

- Clean the PEI print surface with Isopropyl Alcohol (unplugged/cool).

- Insert the SD card and select your file, or send the job via Cura LulzBot Edition.

- Observe the automatic bed leveling sequence to ensure the nozzle touches all four corners correctly.

TROUBLESHOOTING

- Filament Stripping: Check for a clogged nozzle or ensure the idler tension on the extruder is set correctly.

- Bed Leveling Failure: Clean the nozzle tip of any residual plastic to ensure accurate electrical contact during the leveling routine.

Quick Reference Guide: Bemato KB-36 / Grizzly 17" Wood Bandsaw

SAFETY FIRST (READ BEFORE USE)

- Blade Guard Adjustment: Adjust the upper blade guide to be no more than 1/4" above the material being cut.

- PPE: Always wear safety glasses; hearing protection is recommended for extended use.

- Hand Safety: Use a push stick for small pieces to keep fingers away from the blade.

- Wait for Stop: Never back a piece of wood or metal out of a cut while the blade is moving.

BASIC OPERATION

- Preparation: Check that the blade is properly tensioned and tracking in the center of the wheels.

- Guide Setup: Adjust the guide height for your specific workpiece thickness.

- Dust/Coolant: Turn ON the dust collection (wood) or coolant pump (metal/Bemato) if applicable.

- Cutting: Press the START button and feed material slowly, avoiding excessive side pressure.

TROUBLESHOOTING

- Blade Wandering: Check blade tension and ensure the guide bearings are properly adjusted.

- Excessive Vibration: Stop the machine and check for a dull blade or debris on the wheels.

- Motor Bogs Down: Reduce the feed rate; do not force the material into the blade.

Quick Reference Guide: Powermatic Metal Drill Press Model 1150

SAFETY FIRST (READ BEFORE USE)

- Chuck Key: NEVER leave the chuck key in the chuck; remove it immediately after use.

- No Gloves: Do not wear gloves while operating a rotating spindle to prevent entanglement.

- Secure Work: Always clamp the workpiece or use a drill press vise; never hold metal by hand.

- Eye Protection: Safety glasses are mandatory due to high-speed metal chips.

BASIC OPERATION

- Speed Selection: Adjust the variable speed dial ONLY while the motor is running.

- Tooling: Insert the drill bit and tighten securely using all three holes in the chuck.

- Table Setup: Position the table at the correct height and lock all clamping levers.

- Drilling: Apply steady pressure; use cutting oil for steel to prevent overheating.

TROUBLESHOOTING

- Bit Slipping: Ensure the chuck is clean and the bit is tightened properly with the key.

- Smoke at Tip: Reduce spindle speed and apply more lubrication/cooling.

- Won't Start: Check the emergency stop and the main power disconnect.

Quick Reference Guide: BEMATO Vertical Bandsaw KB-36

SAFETY FIRST (READ BEFORE USE)

- Blade Welder: Only trained personnel should use the integrated blade welder.

- Blade Tension: Check tension before every shift; do not over-tighten.

- Clearance: Keep the work table clear of off-cuts to prevent them from wedging against the blade.

BASIC OPERATION

- Speed Range: Select the appropriate speed range (High/Low) based on the material type.

- Blade Guides: Position the guides as close to the work as possible to prevent blade deflection.

- Feeding: Use the miter gauge or a pusher for straight cuts in thick metal.

- Clean up: Use a brush to remove metal chips; never use your bare hands or compressed air near the guides.

TROUBLESHOOTING

- Slow Cutting: Inspect blade teeth for dullness or missing teeth; replace if necessary.

- Coolant Failure: Check the reservoir level and ensure the nozzle is not clogged with chips.

- Squealing: Check the drive belt tension and inspect the pulley alignment.

Quick Reference Guide: Bambu Lab H2D

SAFETY FIRST (READ BEFORE USE)

- Automated Motion: This printer operates at very high speeds; keep the enclosure closed during operation to avoid injury.

- Heat Hazard: The nozzle and heatbed reach high temperatures; allow the machine to cool before removing parts or performing maintenance.

- Ventilation: Ensure proper airflow, especially when printing materials that emit fumes.

- Supervision: Monitor the first layer of every print to ensure proper adhesion and prevent "spaghetti" failures.

BASIC OPERATION

Power-On Sequence:

- Flip the power switch on the rear of the unit.

- Wait for the system to initialize and the touch screen to load.

- Ensure the build plate is clean and properly seated on the magnetic base.

Running a Job:

- Slice your model using Bambu Studio.

- Send the file via Wi-Fi or load it onto the MicroSD card.

- Select the file from the "Files" menu and press Print.

- The machine will perform a self-calibration and bed-leveling sequence before starting.

TROUBLESHOOTING

- First Layer Issues: Check that the correct build plate type is selected in the slicer and that the plate is clean.

- Filament Jam: Inspect the extruder and nozzle for clogs; use the "Unclog" utility in the maintenance menu.

- Connection Error: Restart the machine and check your network settings if the printer does not appear in the software.

Quick Reference Guide: Ultimaker S3 Dual-extrusion

SAFETY FIRST (READ BEFORE USE)

- Dual Nozzles: Both nozzles can be hot simultaneously; use caution when working near the print head.

- Pinch Points: Do not reach into the printer while the gantry or build plate is in motion.

- Material Safety: Use only approved Ultimaker or compatible materials to prevent clogging and damage.

BASIC OPERATION

- Power On: Switch the power button on the back of the machine.

- Load Material: Use the touchscreen to select "Material/PrintCore" and follow the prompts to load filament into Extruder 1 and/or Extruder 2.

- Calibration: The S3 uses active bed leveling; ensure the build plate is free of old plastic before starting.

- Printing: Insert a USB drive or send the job via Ultimaker Digital Factory.

TROUBLESHOOTING

- PrintCore Not Recognized: Ensure the PrintCore is pushed firmly into the print head and the latch is closed.

- Under-extrusion: Check for knots in the filament spool or a dirty feeder gear.

- Cloud Connection: Verify the printer is linked to your Ultimaker account for remote monitoring.

Quick Reference Guide: Haas UMC-500

SAFETY FIRST (READ BEFORE USE)

- Enclosure Safety: Never bypass or disable the door safety interlocks.

- Eye Protection: Always wear safety glasses when the machine is powered on to protect against flying chips and coolant splash.

- Tooling Safety: Ensure all tools are securely seated in the tool changer and that pull studs are tightened to the correct specification.

- 5-Axis Motion: Be aware that the table (B and C axes) can move simultaneously with the spindle; ensure all workholding and parts clear the machine casting during rotation.

- Clearance Check: Always "dry run" new programs with the Rapid Override turned down to check for potential tool crashes.

BASIC OPERATION

Power-On Sequence:

- Turn the main breaker (located on the rear of the machine) to the ON position.

- Press the Power Up/Restart button on the control pendant.

- The machine will automatically home all axes, including the rotary table.

- Ensure the air pressure is at the required level and the coolant is flowing correctly before operating.

Setting Offsets:

- Work Offset: Use the jog handle and a probe or edge finder to locate the part's X, Y, and Z coordinates; input these into the G54-G59 tables.

- Tool Offset: Use the automatic tool setter (if equipped) to define the length and diameter of each tool.

Running a Job:

- Upload your program via USB or network connection.

- Verify the program using Graphics mode on the screen.

- Select Memory mode, ensure the lid is closed, and press Cycle Start.

TROUBLESHOOTING

- Machine Will Not Move: Check if the Emergency Stop is pressed or if the door is open.

- Alarm Active: Read the alarm code on the screen and refer to the machine's internal help menu.

- Coolant/Water Flow Alarm: Check the reservoir level and ensure hoses are not kinked.

Quick Reference Guide: ShopBot CNC Milling Table

SAFETY FIRST (READ BEFORE USE)

- Constant Supervision: NEVER leave the CNC unattended while it is running.

- Personal Safety: Tie back long hair, remove loose clothing, and avoid dangling jewelry to prevent entanglement with moving axes.

- Emergency Stops: Locate the three red emergency stop buttons before starting any job.

- Eye and Ear Protection: Always wear safety glasses and hearing protection when the machine is in operation.

- Debris Management: Ensure the work area is clear of tools and debris that could interfere with gantry movement.

BASIC OPERATION

Power-On Sequence:

- Clear the bed of all obstructions and ensure the gantry moves freely.

- Turn ON the main power switch on the control box.

- Press the Blue Reset Button on the control box to engage the motors.

- Launch the ShopBot control software and run the Spindle Warm-up Routine.

Zeroing and Files:

- Secure your material firmly to the bed using screws or clamps.

- Use the Z-Zero Plate and grounding clip to automatically set the bit height.

- Manually jog the machine to the starting corner of your material and set the X and Y Zero.

- Turn ON the Dust Collector and ensure the extractor is running.

- Load your part file, START the spindle, and click OK to begin.

TROUBLESHOOTING

Machine Will Not Move (Axis Stalled)

- Check E-Stop: Verify that all Emergency Stop buttons (on the control box and handheld pendant) are twisted and released.

- Reset Drive Power: Press the Blue Reset Button on the ShopBot control box. If the internal drivers have "tripped" due to a minor collision, they require a physical reset to engage the motors again.

- Software Connection: Ensure the ShopBot control software is in "Yellow" (Move) mode. If the software shows a "Connection Lost" error, check the USB cable and restart the control box.

- Limit Switches: If the gantry has hit a physical limit switch, you must manually move the machine off the switch using the arrow keys or by physically turning the lead screws (with power OFF).

Spindle Will Not Start

- VFD Error: Check the display on the VFD (Variable Frequency Drive) box. If it shows an error code (e.g., "OL" for Overload), the spindle may have been pushed too hard.

- Interlock Key: Ensure the spindle enable key is in the ON position.

- Warm-up Routine: The spindle may refuse to run at high speeds if the mandatory Warm-up Routine has not been completed for the day.

- Manual Toggle: Ensure the software command to start the spindle has been acknowledged; many setups require you to press "Start" on a physical remote after clicking "OK" on the screen.

Inaccurate Cuts or "Drifting" (Loss of Steps)

- Check Bit Sharpness: A dull bit creates excessive resistance, causing the motors to "lose steps" and drift from the programmed path.

- Secure Workpiece: If the material moves even slightly, the cut will be ruined. Double-check clamps, screws, or vacuum pressure.

- Collet Maintenance: Inspect the collet and nut. If they are dirty or worn, the bit can "creep" out of the spindle during a cut, leading to deeper-than-intended gouges.

- Feed and Speed: If the machine is "screaming" or vibrating, your feed rate may be too high or your spindle RPM too low for the material.

Z-Zero Failed or Inconsistent

- Clean Plate: Ensure the aluminum Z-zero plate is clean and free of sawdust.

- Grounding Clip: Verify the grounding clip is attached to the spindle or the bit itself. If the circuit isn't completed, the bit will crash into the plate.

- Bit Conductivity: Some coated bits or dirty bits may not conduct electricity well; wipe the bit with a cloth before zeroing.

When in Doubt

- Spacebar Stop: In ShopBot software, hitting the Spacebar will immediately stop all motion without killing power to the spindle (useful for minor errors).

- Emergency Stop: Use the Red E-Stop for serious issues (crashes, fire, or broken bits).

- Never Reach In: Never attempt to clear chips or move clamps while the gantry is in motion.

- Ask for help: If the machine makes a "clunking" sound, the drive gears (pinions) may be loose or worn; notify a supervisor immediately.